An earth tube heat exchanger (ETHE) was designed and installed in the initial phase of study. Subsequently, cooling and heating potential of ETHE system was evaluated. Based on the performance of ETHE, an optimum volume of greenhouse that is to be coupled with ETHE system was determined. Finally, ETHE coupled greenhouse was erected and its performance in growing the selected crops was evaluated. In this section, various results obtained in course of study have been presented and discussed objective wise.

Determination of optimum depth for ETHE system

Prior to develop an intended ETHE system, temperatures at 4 different ground depths were measured throughout the year, continuously for two years. These depths were 0 m (0 ft), 0.9 m (3 ft), 1.8 m (6 ft) and 2.7 m (9 ft). Following table (Table 1) indicates the temperature profile inside ground at 10:00 am and 2:00 pm throughout the year.

Table 1 Temperature profile inside ground at 10:00 am and 2:00 pm

| Month | 10:00 am |  |  |  | 2:00 pm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 m | 0.9 m | 1.8 m | 2.7 m | 0 m | 0.9 m | 1.8 m | 2.7 m |
| January | 13.4 | 15.7 | 18.6 | 20.0 | 18.4 | 17.4 | 19.8 | 21.5 |
| February | 17.7 | 17.7 | 18.3 | 19.9 | 21.0 | 19.9 | 20.4 | 22.3 |
| March | 24.7 | 20.9 | 20.9 | 21.5 | 28.7 | 23.7 | 23.3 | 22.5 |
| April | 31.0 | 25.8 | 25.4 | 25.2 | 35.7 | 28.1 | 27.0 | 25.7 |
| May | 38.0 | 29.9 | 28.6 | 28.2 | 42.8 | 30.5 | 29.0 | 28.6 |
| June | 38.4 | 34.7 | 34.4 | 30.4 | 43.7 | 33.6 | 31.2 | 30.5 |
| July | 36.2 | 34.0 | 32.0 | 31.2 | 41.1 | 34.9 | 32.4 | 31.5 |
| August | 35.0 | 32.6 | 31.8 | 31.1 | 38.2 | 33.4 | 32.5 | 31.8 |
| September | 33.3 | 30.9 | 30.7 | 30.7 | 35.7 | 32.0 | 31.9 | 31.8 |
| October | 30.2 | 29.2 | 29.6 | 29.3 | 34.1 | 29.9 | 30.0 | 29.8 |
| November | 19.1 | 21.9 | 23.6 | 24.2 | 23.5 | 23.1 | 25.4 | 24.9 |
| December | 14.8 | 18.3 | 20.3 | 21.4 | 19.8 | 19.9 | 21.5 | 22.4 |

Fig.1 and 2 present temperature profile inside ground during summer and winter months, respectively.  Mean monthly ambient temperatures at Abohar during January (winter month) was found as 14.6°C whereas the ground temperatures at  0.9 m, 1.8 m and 2.7 m depth was found as 16-18°C, 18-20°C and 18-21°C respectively. Ground temperatures at ≥1.8 m depth were significantly higher than the ambient temperature. Similarly, during summer, ground temperatures at ≥1.8 m depth were significantly lower than the ambient temperature. Thus, results underscored the potential of Earth Tube Heat Exchanger (ETHE) for space heating during winter and space cooling during summer months. On the basis of this temperature profile, optimum depth for ETHE system to be installed at ICAR-CIPHET Abohar was determined as ≥1.8 m. Therefore, ETHE system was installed at 2 m of depth.

Fig.1 Temperature profile inside ground during summer months

Fig.2 Temperature profile inside ground during winter months

Pipe material was selected on the basis of heat transfer coefficient. Theoretical analysis was performed to determine diameter and length of the ETHE pipes. The details of ETHE system (Fig.3) installed at ICAR-CIPHET Abohar are as follows.

- Type			: Open and close

- Depth			: 2 m

- Pipe diameter		: 50.8 mm

- Pipe length		: 64 m

- Number of loops	: 10

- Pipe material		: Galvanized iron pipe

- Loop length		: 6 m

- Centre to Centre distance between inlet and outlet pipe: 4.2 m

Fig.3 ETHE system installed at ICAR-CIPHET Abohar

Performance evaluation of ETHE system

After its installation, ETHE system was subjected to performance evaluation during winter and summer months. Air with five different flow rates (4.56, 7.68, 10.8, 13.44 and 25.44 m3/min) was circulated through ETHE system. It was found that ETHE cooled the ambient air (at as high as 49°C) up to 30-34°C irrespective of the flow rate. When air circulated through ETHE pipes at 4.56 m3/min, air temperature dropped up to 30°C. However, when air flow rate was 25.44 m3/min air temperature dropped up to 34°C. Although not significant, outlet temperature was found to be affected by air flow rate. Results also revealed that more the time air gets inside the pipes, more be the cooling and vice versa.

Fig.4 Effect of air flow rate on outlet temperature of ETHE system

Fig.5 depicts the variation in outlet temperature during winter and summer months. It was found that during summer months, when ambient temperature (Ta) was as high as 49°C, ETHE lowered the air temperature (To) up to 34°C (at outlet of the ETHE pipe). Thus, it can be stated that ETHE can be used in space cooling applications. Drop in ambient air temperature after passing through ETHE system during summer months was almost 10°C. On the contrary, gain in ambient air temperature after passing through ETHE system during winter months was only 5°C. Therefore, it can be stated that ETHE system was more efficient in cooling application as compared to heating application.

Summer

Winter

Fig.4 Variation in outlet temperature during winter and summer months

Coefficient of performance (COP) of ETHE system

Coefficient of performance of the earth tube heat exchanger was determined as (ASHRAE 1985).

COP=QoW

Where, Qo= rate of heat lost/gained by air

W = energy input through blower (w)

Energy input through blower was 300 W. The properties of air used in computation related to the temperature of 35°C are given in Table 2.

Table 2 Properties of air at 300 K

| Properties | Value |
| --- | --- |
| Specific heat of air (at 300K) | 1.005 kJ/kg-K |
| Density of air | 1.177 kg/m3 |

Heat lost/gained by air was determined by the air temperature difference at inlet and outlet using following equation.

Qo=mCa(To-Ti)						      …3

Where, m = rate of air mass flow (kg/s)

Ca = specific heat of air (kJ/kg-K)

To = temperature at outlet of the earth tube (°C)

Ti = temperature at inlet of the earth tube (°C)

Results (Table 3 and 4) indicate that COP of ETHE system during cooling application was higher than the COP during heating application which further reveals that ETHE system was more efficient in cooling application as compared to heating application.

Table 3 COP of ETHE system during cooling application

| Flow rate, (m3/min) | Inlet temp (Ti) | Inside temp (Ts) | Outlet temp (To) | Ti-To | Q (kW) | COP |
| --- | --- | --- | --- | --- | --- | --- |
| 4.56 | 41.2 | 27.1 | 32.5 | 8.7 | 0.774 | 2.6 |
| 7.68 | 39.8 | 27.4 | 31.9 | 8.0 | 1.211 | 4.0 |
| 10.8 | 38.0 | 27.0 | 30.6 | 7.4 | 1.581 | 5.3 |
| 13.44 | 41.3 | 27.5 | 30.9 | 10.4 | 2.768 | 9.2 |
| 25.44 | 42.8 | 27.8 | 31.6 | 11.1 | 5.583 | 18.6 |

Table 4 COP of ETHE system during heating application

| Flow rate, (m3/min) | Inlet temp (Ti) | Inside temp (Ts) | Outlet temp (To) | Ti-To | Q (kW) | COP |
| --- | --- | --- | --- | --- | --- | --- |
| 4.56 | 19.7 | 25.0 | 23.7 | 4.0 | 0.358 | 1.2 |
| 7.68 | 18.2 | 23.8 | 22.6 | 4.4 | 0.664 | 2.2 |
| 10.8 | 17.9 | 23.3 | 21.9 | 4.0 | 0.852 | 2.8 |
| 13.44 | 19.7 | 22.7 | 22.6 | 2.9 | 0.778 | 2.6 |
| 25.44 | 18.7 | 21.7 | 21.4 | 2.8 | 1.382 | 4.6 |

Determination of maximum volume of greenhouse to be coupled with ETHE system

Four polytunnels with different volumes (6.44 m3, 12.87 m3, 19.13 m3 and 25.74 m3) were cooled using ETHE system during summer (Fig.5) and effect of ETHE system on temperature inside polytunnels was observed (Fig.6). Similar types of observations were recorded during winter months also. It was observed that when ETHE was ON, the temperature gain (Ti-Ta) was lower (6.7-12.9°C) (Table 5). On the contrary temperature gain was significantly higher (9.6-15.1°C) when ETHE was OFF. Thus, it can be inferred that ETHE showed potential to reduce the inside temperature. However, results also indicated that cooling potential of ETHE system reduced with increase in volume from 6.44 m3 to 25.74 m3. Therefore, determination of maximum (optimum) volume of polyhouse that can be cooled with ETHE system was found essential. In present context, cooling of polyhouse means lowering the inside temperature at least equal to ambient temperature.

Fig.5 Polytunnels used to determine optimum volume of greenhouse

12.87 m3

6.44 m3

25.74 m3

19.13 m3

Fig.6 Effect of polytunnel volume and ETHE on temperature inside polytunnel

Table 5 Effect of ETHE on temperature gain inside polytunnel

| Polytunnel Volume (m3) | Ti-Ta (°C) |  | ∆(Ti-Ta) (°C) |
| --- | --- | --- | --- |
|  | ETHE ON | ETHE OFF |  |
| 6.44 | 6.7 | 10.4 | 3.7 |
| 12.87 | 7.0 | 9.6 | 2.6 |
| 19.13 | 11.2 | 13.5 | 2.3 |
| 25.74 | 12.9 | 15.1 | 2.2 |

Where Ti is inside temperature and Ta is ambient temperature

∆(Ti-Ta): difference between temp gain when ETHE was OFF and temp gain when ETHE was ON

For determining the optimum volume of greenhouse, regression modeling using MINITAB (MINITAB 17) software was used. Neural network modeling using MATLAB (MATLAB R2010a) software was also used to determine the optimum volume of greenhouse. Information on variables and optimal condition is as follows.

- Dependent variable: temperature gain i.e. ∆(Ti-Ta)

- Independent variable: Volume of polytunnel

- Number of observations: 108

- Maximum volume (optimal) condition: ∆(Ti-Ta) ≥1°C

∆(Ti-Ta) was predicted using regression model (produced using MINITAB). It was also predicted using neural network modelling and results ware compared. Algorithm of neural network programme used in the study is given below.

Regression model produced using MINITAB is given by following equation.

∆(Ti-Ta) = 3.567 - 0.0405 V

Observed and predicted values of ∆(Ti-Ta) are given in following Table 6.

Table 6 Observed and predicted values of ∆(Ti-Ta)

| Observed ∆(Ti-Ta) | Predicted ∆(Ti-Ta) |
| --- | --- |
| 3.7 | 3.31 |
| 2.6 | 3.05 |
| 2.3 | 2.8 |
| 2.2 | 2.5 |

Based on optimal condition, optimum volume of greenhouse to be cooled by ETHE was determined as 63.39 m3 say 65 m3.

Development of ETHE coupled greenhouse

Greenhouse was erected after determining its optimum volume. Following assumptions were made prior to development of ETHE coupled greenhouse. Assumptions (for greenhouse cooling) were based on previous observations.

  - Maximum temperature gain by greenhouse: about 10°C

  - Cooling by ETHE: 1-2°C

  - Cooling by ventilation: 3-4°C

  - Cooling by evaporation (foggers): up to 4-5°C

  - Cooling by shading: 3-4°C

Specifications, sketch and photographs (Fig.7 and 8) of ETHE coupled greenhouse developed at ICAR-CIPHET Abohar are given below.

- Volume: 105 m3 which involves 65 m3 (based on ETHE performance) and 40 m3 (based on other cooling arrangements)

- Length: 8 m

- Width: 4 m

- Ridge height: 3.5 m

- Eve height: 2.5 m

- Arch height at ridge: 1 m

- Ventilation: side vents (24% of the total wall surface)

Fig.7 Sketch of ETHE coupled greenhouse

Fig.8 ETHE coupled greenhouse

Extension in the inlet and outlet arrangements of ETHE system

Inlet

Combined 2 inlets (Fig.9) were provided to ETHE system, one for outside air (inlet A) and another for inside air (inlet B). Air with comparatively low temperature was circulated through the system.

Fig.9 Modified inlet of ETHE system

Outlet

Outlet was extended to distribute the air along the length of the structure (Fig.10). Diameter of the distribution point was 3 mm; distance between two consecutive distribution points was 1 ft.

Fig.10 Extended outlet of ETHE system

Empty greenhouse analysis (no crop load)

Empty ETHE coupled greenhouse was provided with four different cooling arrangements, namely ETHE system, cover of black shade net (50% shading intensity), foggers and ventilation (24% side vents).

It was noted that when no cooling arrangement was provided to the structure, temperature gain was 10°C (Fig.10 and Table 7). ETHE alone (without shading, foggers and ventilation) could maintain the temperature gain up to 8.2°C. Thus, it can be inferred that ETHE alone could reduce the greenhouse temperature by 1.8°C.

ETHE + Shade net + side vents 		No ETHE + Shade net + side vents

Fig.10 Performance of ETHE system in lowering inside temperature

Table 7 Performance of different cooling arrangements in lowering inside temperature

| Cooling arrangement | Average inside temp (Tg) | Average ambient temp (Ta) | Tg-Ta (°C) |
| --- | --- | --- | --- |
| No cooling arrangement | 54 | 44 | 10.0 |
| ETHE alone | 53.9 | 45.67 | 8.2 |
| Vents open + shading | 43.9 | 43.00 | 0.9 |
| ETHE + vents open + shading | 45.1 | 44.80 | 0.3 |

Using above mentioned four cooling arrangements, following 16 different combinations were used for cooling the ETHE coupled greenhouse. Temperature gain was noted for each cooling arrangement.

  - ETHE

  - ETHE+SHADENET

  - ETHE+VENTILATION

  - EHTE+FOGGERS

  - EHTE+SHADENET+FOGGERS

  - ETHE+FOGGERS+VENTILATION

  - ETHE+SHADENET+VENTILATION

  - ETHE+SHADENET+FOGGERS+VENTILATION

  - VENTILATION

  - FOGGERS

  - FOGGERS+VENTILATION

  - SHADENET

  - SHADENET+FOGGERS+VENTILATION

  - SHADENET+FOGGERS

  - SHADENET+VENTILATION

  - Without any cooling arrangement

Results revealed that out of 16 arrangements, the most effective cooling arrangement was found to be ETHE+SHADENET+FOGGERS+VENTILATION.  This arrangement lowered the greenhouse temperature by 6.7°C below ambient temperature when ambient temperature varied from 45-50°C.

After empty greenhouse analysis, Tomato, chilli and brinjal crops were grown inside ETHE coupled greenhouse. These crops were grown in two seasons. First season was from May – August 2016 whereas second season was October 2016 – February 2017. ETHE system was operated, as and when required, during crop growing period to provide desirable temperatures inside greenhouse. Crop growth parameters, number of fruits etc. were recorded to understand the effect of ETHE coupled greenhouse on crop growth and health. Following observations indicate the efficacy of ETHE system coupled greenhouse in growing these crops. Only significant results have been mentioned below.

First season:

Tomato and chilli were cultivated in the structure in May 2016 (extreme summer). Significant results were as follows.

  - Tomato and chilli showed 0 and 30% plant mortality, respectively compared to 100 % plant mortality of both seedlings in open field condition. Also, attack of white fly was major concern for crops cultivated under open field condition whereas white fly attack was less severe on the crops inside greenhouse.

Second season:

Tomato and brinjal were cultivated in October 2016. Following results were noticed.

  - Tomato: crop height up to November end

    - Greenhouse = 131.6 cm, Open field = 21.6 cm

  - Average number of flowers at the time of flowering

    - Greenhouse 20; open field 5

  - Average number of fruits at a time

    - Greenhouse 10; open field 3

  - Brinjal:  crop height

    - Greenhouse = 43.4 cm, open field = 16.8 cm

    - Average number of fruits:  Greenhouse = 7; open field = 2.

Cost economics of ETHE coupled greenhouse was determined during present study. Total input cost (non-recurring and recurring) was calculated. Similarly, net annual income from crops cultivated in the structure was determined to calculate the benefit cost ratio. Benefit-cost ratio for this structure was found as 0.69. Following table (Table 8) presents the economic analysis of ETHE coupled greenhouse erected at ICAR-CIPHET Abohar.

Table 8 Cost-economics of ETHE coupled greenhouse

| Cost |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | Non-recurring cost (fixed cost) |  |  |  |  |
|  | Particular | Amount (Rs) | Particular |  | Amount (Rs) |
|  | Polyhouse (frame + cladding) | 26600 | Excavation charges |  | 2250 |
|  | ETHE pipes of GI | 15000 | Blower |  | 2000 |
|  | Drip system | 1000 | Labor |  | 3000 |
|  | Total |  |  |  | 49850 |
| 2. | Fixed cost per annum |  |  |  |  |
|  | Life of the structure is considered as 10 years. Therefore Fixed cost per annum is |  |  | Rs. 4985 |  |
| 3. | Recurring cost |  |  |  |  |
|  | Cost of tomato seeds, fertilizer, water, labour etc. |  |  | Rs. 1700 (1st crop) Rs. 1500 (2nd crops) |  |
|  | Electricity consumption (1 hp blower) |  |  | Rs. 630 (1st crop) Rs. 300 (2nd crops) |  |
|  | Total recurring cost for two crops |  |  | Rs. 4130 (2 crops) |  |
|  | Total cost per annum (2 + 3) |  |  | Rs. 9115 |  |
| Benefit |  |  |  |  |  |
| 1. | Total crop production |  |  |  |  |
|  | Tomato crop 1 (summer crop) |  |  | 512 kg (@15 Rs/kg) |  |
|  | Tomato crop 2 (winter crop) |  |  | 640 kg (@ 12 Rs/kg) |  |
| 2. | Total income |  |  |  |  |
|  | Tomato crop 1 (summer crop) |  |  | Rs.7680 |  |
|  | Tomato crop 2 (winter crop) |  |  | Rs. 7680 |  |
|  | Total income |  |  | Rs. 15360 |  |
| 3. | Net benefit per annum |  |  | Rs. 6245 |  |
| 4. | Benefit : cost ratio |  |  | 0.69 |  |